# Gas Leakage Detection System (Arduino Based)

# 1. Introduction

This project is an Arduino-based Gas Leakage Detection System designed to detect harmful gases and prevent accidents. It provides real-time alerts using LED, buzzer, LCD display, and automatically activates an exhaust fan.

# 2. Objectives

- Detect gas leakage in real-time
- Provide visual and audio alerts
- Automatically reduce gas concentration using a fan
- Ensure safety in homes and industries

# 3. Components Used

Arduino Uno

MQ-5 Gas Sensor

16x2 LCD with I2C Module

LED

Buzzer

Relay Module

12V DC Exhaust Fan

12V Adapter

Breadboard

Jumper Wires

# 4. Working Principle

The MQ-5 gas sensor detects gases like LPG and methane. It sends analog signals to the Arduino. The Arduino processes the data and compares it with predefined threshold values. If the gas level is high, the system triggers alerts and activates the fan.

# 5. Component Functioning

Arduino: Controls the entire system and makes decisions.

MQ-5 Sensor: Detects gas presence and sends signals.

LED: Provides visual indication.

Buzzer: Gives sound alert.

LCD: Displays gas level and system status.

Relay: Controls the fan safely.

Fan: Removes harmful gas.

Power Supply: Provides energy to system.

# 6. Circuit Connections

Sensor: VCC→5V, GND→GND, A0→A0
LCD: VCC→5V, GND→GND, SDA→A4, SCL→A5
LED: Pin 7→LED→GND
Buzzer: Pin 8→Buzzer→GND
Relay: IN→Pin 9, VCC→5V, GND→GND
Fan: Connected via relay and adapter

# 7. Advantages

- Improves safety
- Low cost
- Easy to use
- Automatic response system
- Can be upgraded

# 8. Conclusion

This system is an effective and low-cost solution for detecting gas leakage and preventing accidents.